# MPC-502

# August 19, 2015

Project Title

Experimental and Computational Study of Self-Consolidating Concrete for Prestressed Bridge Girders

University

South Dakota State University

Principal Investigators

Junwon Seo, PhD, PE, M.ASCE

Assistant Professor

Department of Civil and Environmental Engineering

South Dakota State University

316 Crothers Engineering Hall, Box 2219

Brookings, SD 57007

Phone: 605-688-5226

Email: junwon.seo@sdstate.edu

Nadim Wehbe, PhD, PE, F.ACI

Professor and Department Head

Department of Civil and Environmental Engineering

South Dakota State University

212 Crothers Engineering Hall, Box 2219

Brookings, SD 57007

Phone: (605) 688‐4291

Email: nadim.wehbe@sdstate.edu

Research Needs

Self-Consolidating Concrete (SCC) consisting of water, cement, large portion of fine and coarse aggregates, and chemical admixtures is a highly flowable, non-segregating concrete. SCC was initially developed in the 1980’s [1] by concrete researchers at the University of Tokyo in Japan. SCC has significantly different characteristics compared to Conventional Concrete (CC) [1-7]. Specifically, SCC is normally characterized by a slump of 26 to 28 inch in diameter [1] that is different from CC as shown in Figure 1. It means that SCC has its high flowability related to workability that enables to spread into highly dense reinforcement areas under its self-weight and to fill any small voids without mechanical vibration. As a result of the high workability, SCC provides additional benefits as follows [3]:

- Low noise-level in the plants and construction sites

- Eliminated problems associated with vibration

- Less labor involved

- Faster construction

- Improved quality and durability, and

- Higher strength

(a) 				                  (b)

Figure 1 Slump Test: (a) SCC and (b) CC [7].

With these benefits, the use of SCC can result in better construction quality, productivity, and safety [4]; thus, construction contractors, producers and owners who are faced with tougher environmental and stricter safety guidelines, and increased construction costs have attempted to use SCC in many construction applications [3, 6]. Further, SCC has high potential for precast, prestressed concrete industry and for cast-in-place construction in the United States [3]. For example, a National Cooperative Highway Research Program (NCHRP) research project has been initiated to establish SCC bridge girder design and construction guidelines to be added to the AASHTO LRFD Specifications. The South Carolina State Department of Transportation (SCDOT) was to study the use of SCC in drilled shafts [3] and in replacement of four structurally deficient bridges [5]. These studies have helped SCDOT use SCC in the drilled shafts and bridges. Prestressed SCC girders used for the bridges’ replacement can be shown in Figure 2. Also, the Kansas State Department of Transportation (KSDOT) performed a study of the fresh and hardened properties of SCC for use in Kansas prestressed concrete bridge girders. KSDOT built a three-span bridge using SCC in only one span while the remaining two spans were built using CC [3]. The bridge is instrumented and monitored for five years to evaluate its long-term performance. In addition to the field studies, experimental studies for SCC concrete bridge girders have been performed by South Dakota State University (SDSU) in cooperation with the South Dakota DOT (SDDOT) [7]. The flexural behavior of prestressed SCC bridge girders subjected to monotonic and fatigue loading was experimentally examined at the Lohr Structures Laboratory at SDSU. In this testing, three full-scale prestressed bridge girders were made and tested until failure. One girder was constructed using CC as a reference specimen, while the other two girders were constructed using SCC. It was concluded that SCC and CC bridge girders showed similar behavior under monotonic and fatigue loading. Figure 3 shows a sample picture for the representative SCC bridge girder used for the testing at SDSU.

Figure 2 Use of Prestressed SSC Bridge Girders at SCDOT [5].

Figure 3 Testing of Representative Prestressed SCC Bridge Girder at SDSU [7].

From the brief literature review, it can be concluded that relevant studies [1-7] to understand the structural behaviors of prestressed SCC bridge girders and attempt to use SCC in actual prestressed bridge girders based upon individual state bridge design guidelines and construction methods have been completed. However, there has been no specific test data to investigate the material properties (i.e., modulus, shrinkage, and creep) of prestressed SCC bridge girders related to time-dependent characteristics, flexural stiffness change, and prestress losses. Considering these effects is critical for the use of SCC in prestressed bridge girders because the structural performance of prestressed girders is typically related to the material properties associated with time-dependent factors. In particular, the material properties of SCC used by precast prestressed girder suppliers to the Wisconsin Department of Transportation (WisDOT) have not been examined yet. Lacking substantiated data on the material properties, WisDOT has not allowed the use of SCC in prestressed bridge girders in Wisconsin. Even precasters have not been able to consistently mix, deliver and place SCC in prestressed bridge girder construction. Although WisDOT attempted to use SCC in some prestressed girders, the producer had difficulty in maintaining uniformity of the mix, and problems were observed with excessive segregation of wet batches during placement. Hence, widely accepted, uniform recommendations for SCC mix design that achieves desired performance for use in prestressed girders for WisDOT should be developed for ensuring safety in its construction.

Research Objectives

SCC has high potential prestressed bridge construction in Wisconsin, but there is a lack of definitive data on its material properties for WisDOT. The primary objective of the work proposed herein is to develop a Wisconsin specific mix design and its specification for the use of SCC in prestressed girders used on WisDOT bridge projects. To accomplish this objective, this project will test several SCC samples for examining the following characteristics:

- Material Properties

- Time-Dependent Characteristics

- Prestress Losses

This project will not only help in better understanding the material properties of prestressed SCC girders, but will also provide recommendations related to its design and guidance that can be useful for a full-scale implementation on one of the department’s bridge projects. This implementation will be coordinated with WisDOT Bureau of Structures and a Precast Girder Producer for the design, implementation, testing, monitoring and documentation. Field monitoring-based structural evaluation process will also be included to evaluate the structural performance of a test prestressed SCC bridge girder on the project.

Research Methods

The key objective of this project is to determine the material properties, time-dependent characteristics, prestress losses of SCC required for the design and implementation of a prestressed SCC bridge girder on one of the WisDOT bridge projects. The research methods that will be used to achieve the objectives include three main components.

- Literature Review:

The existing literature related to material properties and structural characteristics of prestressed SCC bridge girders will be reviewed. To seek information on the current state of the art and practice, the research team will use different types of search engines such as Compendex and ASCE Civil Engineering database. Given the SDSU research team’s acquaintance and geographical proximity, Wisconsin and South Dakota bridge engineers with knowledge of and experience with development of a specific mix design and specification for the use of SCC in prestressed girders will be directly contacted for input.

- Experimental Investigation of SCC Material Characteristics:

Experimental matrix considering the variability in some factors such as mixture proportions will be established to determine various material characteristics of SCC in a practical and systematic manner.

- Test SCC Girder Implementing, Monitoring, and Documenting:

A prestressed concrete bridge located in Wisconsin will be selected, and a test girder made with SCC according to new recommendations to be developed from this project will be placed in the selected bridge. The implementation of an optimized monitoring system, which includes multiple sensors attached to the critical regions of the bridge will be made in cooperation with the WisDOT Bureau of Structures. Specifically, the SCC girder and a CC girder as a reference girder will be instrumented to capture time-dependent prestress losses, transfer length, and the flexural and shear responses of the girders as follows:

- Embedded strain gages

- External sensors, such as strain gages and LVDTs

Expected Outcomes

The benefit derived from this project will be the development of standard SCC mix design that achieves desired structural performance for use in prestressed girders for WisDOT. This project will result in new recommendations for prestressed SCC mix design that are comparable to the current AASHTO and PCI bridge design codes. The recommendations will allow for more confidence in the use of SCC in prestresed girders in Wisconsin. To be of utmost benefit to the WisDOT, a document summarizing standard SCC mix proportions to build prestressed girders and all test results for their time-dependent characteristics and prestress losses will be provided. The final report will be submitted in an electronic PDF format and eight hard copies of the final report will be delivered to the WHRP by the contract deadline.

Relevance to Strategic Goals

The anticipated products are related to the following strategic goals: State of Good Repair, Safety, and Economic Competiveness.

Educational Benefits

One graduate student will be involved in this project so that the student will secure invaluable research experience and knowledge in the use of SCC in prestressed bridge girders. A guest lecture containing the introduction of the SCC and its use in prestressed bridge girders will be established for SDSU CEE 769 Bridge Engineering.

Work Plan

Task 1: Literature review of the state of the art and practice in SCC for prestressed bridge girders

Task 2: Prestressed SCC girder supplier identification

Task 3: Experimental examination of SCC material characteristics

Task 4: SCC design recommendation and guidance development

Task 5: Test SCC Wisconsin bridge girders implementing, monitoring, and documenting

Task 6: Develop and validate a computational model to simulate anticipated behaviors of the tested bridge.

Task 7: Perform a parametric study to investigate live-load performance sensitivity for the SCC bridges.

Task 8: Final report

Project Cost

Total Project Costs:   $100,000

MPC Funds Requested:  $30,000

Matching Funds:  $ 70,000                       Source of Matching Funds: Wisconsin DOT

TRB Keywords

Self-Consolidating Concrete, Prestressed Bridge Girders, Experimental Testing, Field Monitoring, Implementation, Structural Performance Evaluation, Computational Model, Parametric Study.

References

- Bartos, P.J.M., “Measurement of Key Properties of Self-Compacting Concrete,” CEN/STAR PNR Workshop, June 2000, (http://bativille.cstb.fr/CenStarWS/Measurement_key_properties.pdf).

- Naik, T.R. and Kumar, R. (2001). “Self-compacting concrete (SCC) of self-leveling concrete (SLC),” Report No. CBU-2001-24, Milwaukee, WI

- Ouchi, M., Nakamura, S., Osterberg, T., Hallberg, S.-E., and Lwin, M. (2003). “Applications of self-compacting concrete in Japan, Europe and the United States.” ISHPC Report, http://www.fhwa.dot.gov/bridge/scc.cfm

- Koehler, E.P. and Fowler, D.W. (2007). “Inspection manual for self-consolidating concrete in precast members.” TxDOT Project 0-5134, Austin, TX.

- Mallela, J., Littleton, P., and Hoffman, G. (2010). “South Carolina demonstration project: Black River Bridge replacements on SC 377 in Williamsburg County.” Final Report, Washington, DC

- GRACE Construction Products. (2005). “An introduction to self-consolidating concrete (SCC).” Technical Bulletin, Cambridge, MA

- Wehbe N., Sigl, A., Gutzmer, Z., and Stripling, C. (2009). “Structural performance of prestressed self-consolidating concrete bridge girders made with limestone aggregates.” SDDOT Report SD2007-04, Pierre, South Dakota